SITE: RS01

EASTING: 559434

NORTHING: 4894296

ELEV: 490

SLOPE: 41

ASPECT: 200

MODELED RADIATION REGIME:

J      F      M      A      M      J      J      A      S      O      N      D

TOTAL RADIATION*   0.76   1.43   2.27   3.07   4.58   5.93   6.47   5.17   2.99   1.93   0.97   0.69

BLKD BY TOPO**  0.138  0.135  0.117  0.108  0.105  0.089  0.087  0.088  0.096  0.118  0.138  0.159

BLKD BY CANOPY**  0.679  0.674  0.703  0.734  0.672  0.651  0.649  0.692  0.760  0.705  0.671  0.660

*DIFFUSE & DIRECT ON SLOPED SURFACE, MJ/m².day

(INCLUDING CLOUDINESS, TOPOGRAPHY, AND CANOPY)

**PROPORTIONS OF TOTAL INCOMING RADIATION AT SURFACE

CANOPY: Sparse, moderately high. DISCONTINUOUS, with numerous open and shaded areas.

SURFACE: Scrubby bushes, grasses, small felled timber.

SENSOR: 1.0 m and .8 m mercury bulb thermometers.

GENERAL SITE CHARACTERISTICS:

Site is on southwest-facing steep slope with open views of Blue River Res. and ridges to the

south and west.  Canopy coverage very discontinuous, from severe insolation blockeage to no

direct insolation blockeage for every month of the year.  Most blockeage from canopy, not

topography.  Direct sun in late afternoon for every month.

DATES OF OPERATION: 4/70-7/95

SITE USED FOR MAPPING? no

SITE: RS02

EASTING: 560513

NORTHING: 4896132

ELEV: 490

SLOPE: 22

ASPECT: 285

MODELED RADIATION REGIME:

J      F      M      A      M      J      J      A      S      O      N      D

TOTAL RADIATION*   0.81   1.08   1.51   2.31   3.75   4.93   5.18   4.13   1.98   1.22   0.87   0.76

BLKD BY TOPO**  0.215  0.123  0.098  0.088  0.076  0.070  0.063  0.068  0.071  0.098  0.175  0.257

BLKD BY CANOPY**  0.687  0.790  0.829  0.828  0.775  0.754  0.757  0.779  0.854  0.833  0.738  0.647

*DIFFUSE & DIRECT ON SLOPED SURFACE, MJ/m².day

(INCLUDING CLOUDINESS, TOPOGRAPHY, AND CANOPY)

**PROPORTIONS OF TOTAL INCOMING RADIATION AT SURFACE

CANOPY: High/low mix, sparse forest, DISCONTINUOUS CANOPY TYPE; mostly closed, with two significant

open areas.

SURFACE: Soft forest floor, thick low ferns and shrubs, amongst fallen tree trunks.

SENSOR: 1.0, 2.25 m mercury bulb thermometer and thermister shielded above with PVC.

GENERAL SITE CHARACTERISTICS:

In sparse forest of mixed-aged trees.  Near total attenuation of direct radiation from Oct. through

Feb.  In summer, direct radiation just after midday for 2-3 hours.

DATES OF OPERATION: 5/70-present

SITE USED FOR MAPPING? no

SITE: RS03

EASTING: 567175

NORTHING: 4900777

ELEV: 945

SLOPE: 5

ASPECT: 315

MODELED RADIATION REGIME:

J      F      M      A      M      J      J      A      S      O      N      D

TOTAL RADIATION*   0.37   0.60   0.85   1.43   2.53   2.38   2.61   2.86   1.28   0.70   0.44   0.38

BLKD BY TOPO**  0.060  0.054  0.047  0.042  0.037  0.029  0.029  0.031  0.034  0.047  0.062  0.080

BLKD BY CANOPY**  0.892  0.903  0.917  0.907  0.865  0.894  0.891  0.865  0.918  0.920  0.895  0.872

*DIFFUSE & DIRECT ON SLOPED SURFACE, MJ/m².day

(INCLUDING CLOUDINESS, TOPOGRAPHY, AND CANOPY)

**PROPORTIONS OF TOTAL INCOMING RADIATION AT SURFACE

CANOPY: Shady, thick, low canopy.  Much low evergreen vegetation.  CONTINUOUS, CLOSED CANOPY TYPE.

SURFACE: Thick vegetation (grasses and shrubs), and fallen tree trunks.

SENSOR: 1.0 and 2.35 m mercury bulb thermometers.

GENERAL SITE CHARACTERISTICS:

Site is tucked away in thick grove in thick forest.  Little direct sun except during mid-afternoon

in summer.  Site has very little topographic attenuation but great amount of canopy attenuation.

Directly south just beyond near trees is a large clearing (edge effects?).  Site is defunct;

canopy photo a best estimate.

DATES OF OPERATION: 4/70-7/95

SITE USED FOR MAPPING? yes

SITE: RS04

EASTING: 568985

NORTHING: 4902368

ELEV: 1310

SLOPE: 27

ASPECT: 270

MODELED RADIATION REGIME:

J      F      M      A      M      J      J      A      S      O      N      D

TOTAL RADIATION*   0.68   1.07   1.52   1.97   2.73   3.63   4.01   2.59   2.00   1.40   0.72   0.63

BLKD BY TOPO**  0.151  0.124  0.096  0.076  0.087  0.060  0.067  0.067  0.086  0.121  0.118  0.173

BLKD BY CANOPY**  0.753  0.789  0.828  0.854  0.834  0.819  0.812  0.864  0.853  0.807  0.795  0.732

*DIFFUSE & DIRECT ON SLOPED SURFACE, MJ/m².day

(INCLUDING CLOUDINESS, TOPOGRAPHY, AND CANOPY)

**PROPORTIONS OF TOTAL INCOMING RADIATION AT SURFACE

CANOPY: High and sparse; CONTINUOUS, CLOSED CANOPY TYPE with some small clearings direcly overhead.

SURFACE: Soft forest floor, pine needles and light vegetation.

SENSOR: Mercury bulb thermometers and open thermisters shielded above with PVC; varied in height

from 1.0 to 4.0 m during period of operation.

GENERAL SITE CHARACTERISTICS:

Highest site in the HJ Andrews, facing due west on a steep slope.  Little topographic attenuation.

No significant periods of uninterrupted sunlight.

DATES OF OPERATION: 6/70-present

SITE USED FOR MAPPING? no

SITE: RS05

EASTING: 563769

NORTHING: 4896670

ELEV: 880

SLOPE: 12

ASPECT: 10

MODELED RADIATION REGIME:

J      F      M      A      M      J      J      A      S      O      N      D

TOTAL RADIATION*   0.65   0.70   0.97   1.89   2.66   2.61   2.79   3.02   1.88   0.86   0.61   0.73

BLKD BY TOPO**  0.127  0.043  0.027  0.021  0.023  0.019  0.019  0.016  0.020  0.035  0.077  0.212

BLKD BY CANOPY**  0.803  0.890  0.909  0.882  0.863  0.887  0.887  0.862  0.884  0.904  0.856  0.719

*DIFFUSE & DIRECT ON SLOPED SURFACE, MJ/m².day

(INCLUDING CLOUDINESS, TOPOGRAPHY, AND CANOPY)

**PROPORTIONS OF TOTAL INCOMING RADIATION AT SURFACE

CANOPY: Mixed high/medium height, sparse; CONTINUOUS, CLOSED CANOPY TYPE.

SURFACE: Soft forest floor, bed of pine needles and leaves.

SENSOR: 1.0, 2.0 m mercury bulb thermometer and thermister shielded above with PVC.

GENERAL SITE CHARACTERISTICS:

In sparse to moderate forest of mostly older trees, on a broad flat area on the north-facing

side of Lookout Ridge.  Virtually no topographic attenuation in summer, but shady site due almost

exclusively to presence of canopy.

DATES OF OPERATION: 1/71-present

SITE USED FOR MAPPING? yes

SITE: RS07

EASTING: 560023

NORTHING: 4895655

ELEV: 460

SLOPE: 19

ASPECT: 1

MODELED RADIATION REGIME:

J      F      M      A      M      J      J      A      S      O      N      D

TOTAL RADIATION*   0.71   0.86   1.13   1.71   2.39   2.32   2.31   2.31   1.46   0.99   0.78   0.57

BLKD BY TOPO**  0.230  0.115  0.067  0.053  0.050  0.041  0.038  0.038  0.050  0.094  0.209  0.201

BLKD BY CANOPY**  0.665  0.791  0.846  0.851  0.838  0.870  0.877  0.853  0.861  0.824  0.697  0.695

*DIFFUSE & DIRECT ON SLOPED SURFACE, MJ/m².day

(INCLUDING CLOUDINESS, TOPOGRAPHY, AND CANOPY)

**PROPORTIONS OF TOTAL INCOMING RADIATION AT SURFACE

CANOPY: Low and dense; CONTINUOUS, CLOSED CANOPY TYPE.

SURFACE: Thick shrubs and ferns.

SENSOR: .65, 1.0 m mercury bulb thermometers and thermister shielded above with PVC.

GENERAL SITE CHARACTERISTICS:

Site is in a dense forest of mixed-aged trees, on a steep northwest-facing slope just above

CS2MET. Canopy relatively open due north. Not far from Lookout Creek, so possible is susceptible

to cold-air drainage issues.

DATES OF OPERATION: 5/70-present

SITE USED FOR MAPPING? yes

SITE: RS10

EASTING: 562474

NORTHING: 4897908

ELEV: 610

SLOPE: 6

ASPECT: 170

MODELED RADIATION REGIME:

J      F      M      A      M      J      J      A      S      O      N      D

TOTAL RADIATION*   0.45   0.87   1.38   1.81   1.90   1.63   1.56   2.12   1.93   1.20   0.62   0.50

BLKD BY TOPO**  0.040  0.039  0.032  0.023  0.024  0.022  0.016  0.019  0.020  0.029  0.048  0.076

BLKD BY CANOPY**  0.878  0.870  0.875  0.889  0.902  0.929  0.937  0.906  0.888  0.876  0.862  0.842

*DIFFUSE & DIRECT ON SLOPED SURFACE, MJ/m².day

(INCLUDING CLOUDINESS, TOPOGRAPHY, AND CANOPY)

**PROPORTIONS OF TOTAL INCOMING RADIATION AT SURFACE

CANOPY: Low, dense; CONTINUOUS, CLOSED CANOPY TYPE.

SURFACE: Mossy, soft bed of pine needles and decomposing logs.

SENSOR: 1.0, 1.9, 2.0 m mercury bulb thermometer and thermister shielded above with PVC.

GENERAL SITE CHARACTERISTICS:

In thick forest of mostly young-middle-aged trees, low canopy. Very little radiation ever reaches

site; canopy very thick.  On a south-facing slope. Virtually no topographic shading during any

months.

DATES OF OPERATION: 4/71-present

SITE USED FOR MAPPING? yes

SITE: RS12

EASTING: 570409

NORTHING: 4897130

ELEV: 1007

SLOPE: 11

ASPECT: 282

MODELED RADIATION REGIME:

J      F      M      A      M      J      J      A      S      O      N      D

TOTAL RADIATION*   0.76   1.27   1.83   2.70   3.63   4.29   4.42   4.00   2.74   1.51   0.90   0.69

BLKD BY TOPO**  0.210  0.186  0.141  0.116  0.116  0.109  0.096  0.095  0.103  0.134  0.201  0.220

BLKD BY CANOPY**  0.733  0.763  0.807  0.810  0.787  0.791  0.801  0.804  0.823  0.816  0.748  0.724

*DIFFUSE & DIRECT ON SLOPED SURFACE, MJ/m².day

(INCLUDING CLOUDINESS, TOPOGRAPHY, AND CANOPY)

**PROPORTIONS OF TOTAL INCOMING RADIATION AT SURFACE

CANOPY: Sparse and high; CONTINUOUS, CLOSED CANOPY TYPE

SURFACE: Sparse small bushes and grass.

SENSOR: 1.0 to 3.0 m mercury bulb thermometers and thermister shielded above with PVC.

GENERAL SITE CHARACTERISTICS:

In a moderate forest of middle to older-aged trees.  Near Lookout Creek at its upper reaches,

just uphill from TSLOOK; susceptible to cold-air drainage.  Significant topographic attentuation

during winter months.  In summer, direct radiation breifly during mid-afternoon.

DATES OF OPERATION: 7/71-present

SITE USED FOR MAPPING? no

SITE: RS15

EASTING: 560821

NORTHING: 4895184

ELEV: 760

SLOPE: 33

ASPECT: 350

MODELED RADIATION REGIME:

J      F      M      A      M      J      J      A      S      O      N      D

TOTAL RADIATION*   0.53   0.97   1.25   1.56   2.28   2.44   2.45   2.13   1.35   1.09   0.68   0.47

BLKD BY TOPO**  0.568  0.326  0.188  0.110  0.100  0.078  0.070  0.073  0.111  0.246  0.514  0.548

BLKD BY CANOPY**  0.356  0.606  0.754  0.822  0.801  0.832  0.839  0.828  0.827  0.700  0.418  0.377

*DIFFUSE & DIRECT ON SLOPED SURFACE, MJ/m².day

(INCLUDING CLOUDINESS, TOPOGRAPHY, AND CANOPY)

**PROPORTIONS OF TOTAL INCOMING RADIATION AT SURFACE

CANOPY: Sparse and high; CONTINUOUS, CLOSED CANOPY TYPE.

SURFACE: Soft bed of pine needles and decaying logs.

SENSOR: 1.0, 1.8 m mercury bulb thermometers.

GENERAL SITE CHARACTERISTICS:

Site is on a very steep north-facing slope; great solar attenuation in winter due to topographic

shading to south.  Midday in summer site receives the most direct insolation of the year.

Site is defunct; canopy photos a best estimate.

DATES OF OPERATION: 7/72-10/94

SITE USED FOR MAPPING? no

SITE: RS16

EASTING: 560694

NORTHING: 4895527

ELEV: 640

SLOPE: 29

ASPECT: 202

MODELED RADIATION REGIME:

J      F      M      A      M      J      J      A      S      O      N      D

TOTAL RADIATION*   0.78   1.10   1.85   2.65   2.52   2.67   2.46   2.90   2.97   1.75   0.96   0.72

BLKD BY TOPO**  0.316  0.180  0.143  0.112  0.111  0.113  0.090  0.098  0.109  0.166  0.305  0.416

BLKD BY CANOPY**  0.635  0.773  0.782  0.795  0.836  0.856  0.879  0.845  0.789  0.758  0.649  0.535

*DIFFUSE & DIRECT ON SLOPED SURFACE, MJ/m².day

(INCLUDING CLOUDINESS, TOPOGRAPHY, AND CANOPY)

**PROPORTIONS OF TOTAL INCOMING RADIATION AT SURFACE

CANOPY: Low, dense; CONTINUOUS, CLOSED CANOPY TYPE.

SURFACE: Fallen logs and thick vegetation (rhododendron bushes).

SENSOR: 1.0, 1.8 m mercury bulb thermometers.

GENERAL SITE CHARACTERISTICS:

Site is on a steep west-facing slope; recieves direct solar radiation mid-mornings in spring and

fall.  Vegetation is thick and mixture of rhododendrons and evergreens.

Site is defunct; canopy photos a best estimate.  Extremely unsure of fisheye photo accuracy.

DATES OF OPERATION: 7/72-10/94

SITE USED FOR MAPPING? no

SITE: RS17

EASTING: 560767

NORTHING: 4896468

ELEV: 490

SLOPE: 14

ASPECT: 315

MODELED RADIATION REGIME:

J      F      M      A      M      J      J      A      S      O      N      D

TOTAL RADIATION*   0.61   0.89   1.40   2.21   2.89   3.20   3.26   3.03   2.10   1.09   0.66   0.58

BLKD BY TOPO**  0.103  0.075  0.066  0.040  0.039  0.034  0.030  0.031  0.042  0.064  0.092  0.134

BLKD BY CANOPY**  0.794  0.834  0.843  0.842  0.834  0.848  0.853  0.841  0.843  0.849  0.817  0.765

*DIFFUSE & DIRECT ON SLOPED SURFACE, MJ/m².day

(INCLUDING CLOUDINESS, TOPOGRAPHY, AND CANOPY)

**PROPORTIONS OF TOTAL INCOMING RADIATION AT SURFACE

CANOPY: High, sparse; CONTINUOUS, CLOSED CANOPY TYPE with one significant opening directly overhead.

SURFACE: Soft forest floor, pine needles and ferns.

SENSOR: .6, 1.0 m mercury bulb thermometers.

GENERAL SITE CHARACTERISTICS:

Site is on northwest-facing slope not far from Lookout Creek; cold-air drainage may affect

temperature regimes.  In a thick forest with high canopy, sparse to the west.

Site is defunct; canopy photo a best estimate.  Unsure of fisheye photo accuracy.

DATES OF OPERATION: 6/72-7/95

SITE USED FOR MAPPING? yes

SITE: RS20

EASTING: 559997

NORTHING: 4896597

ELEV: 683

SLOPE: 34

ASPECT: 180

MODELED RADIATION REGIME:

J      F      M      A      M      J      J      A      S      O      N      D

TOTAL RADIATION*   0.43   0.79   1.55   2.85   3.17   3.38   3.68   4.00   3.42   1.41   0.54   0.41

BLKD BY TOPO**  0.029  0.027  0.024  0.019  0.022  0.018  0.016  0.015  0.018  0.024  0.028  0.042

BLKD BY CANOPY**  0.871  0.872  0.849  0.810  0.818  0.836  0.836  0.812  0.798  0.849  0.871  0.858

*DIFFUSE & DIRECT ON SLOPED SURFACE, MJ/m².day

(INCLUDING CLOUDINESS, TOPOGRAPHY, AND CANOPY)

**PROPORTIONS OF TOTAL INCOMING RADIATION AT SURFACE

CANOPY: Low and sparse; DISCONTINUOUS CANOPY TYPE.

SURFACE: Pine needles, dirt, and sticks.

SENSOR: 1.0, 2.2 m mercury bulb thermometers and thermister shielded above with PVC.

GENERAL SITE CHARACTERISTICS:

On a steep south-facing slope near the top of the lower Blue River Ridge.  Sparse and discontinuous

canopy to east, south, and west.  Though canopy is thin, attenuation due to it is significant;

very little topographic attenuation due to position near top of ridge.  In a moderate forest

of young to middle-aged evergreens.

DATES OF OPERATION: 1/79-present

SITE USED FOR MAPPING? yes

SITE: RS26

EASTING: 565992

NORTHING: 4901852

ELEV: 1040

SLOPE: 20

ASPECT: 180

MODELED RADIATION REGIME:

J      F      M      A      M      J      J      A      S      O      N      D

TOTAL RADIATION*   0.58   1.12   2.16   2.71   3.13   3.28   3.50   3.71   3.07   1.94   0.78   0.53

BLKD BY TOPO**  0.031  0.025  0.027  0.020  0.020  0.019  0.016  0.015  0.017  0.023  0.030  0.039

BLKD BY CANOPY**  0.834  0.826  0.798  0.826  0.828  0.848  0.851  0.832  0.823  0.798  0.821  0.827

*DIFFUSE & DIRECT ON SLOPED SURFACE, MJ/m².day

(INCLUDING CLOUDINESS, TOPOGRAPHY, AND CANOPY)

**PROPORTIONS OF TOTAL INCOMING RADIATION AT SURFACE

CANOPY: High, thick; CONTINUOUS, CLOSED CANOPY TYPE, though thin to south.

SURFACE: Pine needles, sticks, small plants.

SENSOR: 1.0, 2.0 m mercury bulb thermometers and thermister shielded above with PVC.

GENERAL SITE CHARACTERISTICS:

In thick forest of young, middle, and old-aged trees.  On a slope facing due south, with very

little topographic attenuation thoughout the year.

DATES OF OPERATION: 1/78-present

SITE USED FOR MAPPING? yes

SITE: RS38

EASTING: 565480

NORTHING: 4901379

ELEV: 977

SLOPE: 15

ASPECT: 170

MODELED RADIATION REGIME:

J      F      M      A      M      J      J      A      S      O      N      D

TOTAL RADIATION*   0.61   1.23   2.55   3.90   3.75   3.50   3.76   4.38   4.45   2.31   0.84   0.60

BLKD BY TOPO**  0.034  0.040  0.030  0.027  0.028  0.021  0.022  0.020  0.024  0.034  0.038  0.060

BLKD BY CANOPY**  0.830  0.807  0.761  0.752  0.797  0.841  0.842  0.803  0.742  0.758  0.809  0.805

*DIFFUSE & DIRECT ON SLOPED SURFACE, MJ/m².day

(INCLUDING CLOUDINESS, TOPOGRAPHY, AND CANOPY)

**PROPORTIONS OF TOTAL INCOMING RADIATION AT SURFACE

CANOPY: Thin, low; CONTINUOUS, CLOSED CANOPY TYPE, though site used to be clear-cut.

SURFACE: Leaves, shrubs, and dirt.

SENSOR: 1.0 m mercury bulb thermometers and thermister shielded above with PVC.

GENERAL SITE CHARACTERISTICS:

Site is in south-facing slope amongst young dense evergreen forest.  Historically an unusually

warm site due to thin low canopy, southern exposure, and fact that it used to be clear-cut (has

since grown back).  Canopy appears to act as a sort of magnifier of solar radiation, keeping

site inordinately warm.

DATES OF OPERATION: 6/75-present

SITE USED FOR MAPPING? no

SITE: RS86

EASTING: 559377

NORTHING: 4896330

ELEV: 653

SLOPE: 28

ASPECT: 215

MODELED RADIATION REGIME:

J      F      M      A      M      J      J      A      S      O      N      D

TOTAL RADIATION*   1.58   3.14   6.03   9.05   9.11   8.93   9.70  11.53  10.74   5.45   2.09   1.42

BLKD BY TOPO**  0.018  0.024  0.024  0.016  0.021  0.016  0.015  0.013  0.021  0.022  0.030  0.033

BLKD BY CANOPY**  0.432  0.395  0.319  0.313  0.413  0.521  0.523  0.394  0.268  0.306  0.389  0.414

*DIFFUSE & DIRECT ON SLOPED SURFACE, MJ/m².day

(INCLUDING CLOUDINESS, TOPOGRAPHY, AND CANOPY)

**PROPORTIONS OF TOTAL INCOMING RADIATION AT SURFACE

CANOPY: Open site except for bushy deciduous tree directly overhead and to east; CONTINUOUS,

OPEN CANOPY TYPE.

SURFACE: Fallen leaves and dirt.

SENSOR: 1.0, 1.6, 2.35 m mercury bulb thermometers and thermister shielded above with PVC.

GENERAL SITE CHARACTERISTICS:

Site located at the top of large clear-cut, almost completely open to south.  On a southwest-

facing slope near the top of lower Blue River Ridge.  Horizon and distant ridges clearly visible

from east-south-west.  Direct insolation blocked until noon in summer due to fact that sensor is

located directly to the west of tall bushy deciduous tree.  Almost no topographic attenuation;

virtually all blockeage of insolation due to canopy and surrounding trees.

DATES OF OPERATION: 9/75-present

SITE USED FOR MAPPING? no

SITE: RS89

EASTING: 559227

NORTHING: 4896106

ELEV: 475

SLOPE: 37

ASPECT: 315

MODELED RADIATION REGIME:

J      F      M      A      M      J      J      A      S      O      N      D

TOTAL RADIATION*   0.59   0.94   1.58   3.02   3.28   3.88   4.09   3.49   2.83   1.27   0.73   0.50

BLKD BY TOPO**  0.246  0.207  0.183  0.166  0.135  0.120  0.114  0.114  0.141  0.192  0.305  0.251

BLKD BY CANOPY**  0.656  0.706  0.719  0.662  0.717  0.729  0.733  0.732  0.677  0.715  0.607  0.652

*DIFFUSE & DIRECT ON SLOPED SURFACE, MJ/m².day

(INCLUDING CLOUDINESS, TOPOGRAPHY, AND CANOPY)

**PROPORTIONS OF TOTAL INCOMING RADIATION AT SURFACE

CANOPY: Low, thick evergreen ; CONTINUOUS, CLOSED CANOPY TYPE.

SURFACE: Very bushy vegetation.

SENSOR: .85, 1.0, 2.85 m mercury bulb thermometers and thermister shielded above with PVC.

GENERAL SITE CHARACTERISTICS:

Site is located near bottom of narrow east-west gulley with small creek.  On a very steep

north-facing slope.  Just beyond near trees to north and west is a large open area.  Site

recieves direct sun late on summer afternoons.  Historical temperature trends appear unusually

high.  Just uphill from GSWS10.  Dark and shady every morning of year until noon.

DATES OF OPERATION: 9/75-present

SITE USED FOR MAPPING? no